Reported Symptom(s):

Some PCIe devices not detected with newer firmware: * onboard Ethernet * onboard wi-fi * onboard ASMedia USB controller I initially encountered this problem on kernel version 4.15.0-2; I upgraded to 4.16.5-1 just in case that might fix the problem, but it did not. This is a brand-new motherboard, CPU, and RAM, but so far they test OK with the 0401 BIOS. The other components are well-tested in other computers previously.

| SPECIFICATIONS: | CUSTOMER |
| --- | --- |
| Product Model & S/N: | Crosshair VII Hero Wi-Fi, S/N: 180220540301484 |
| BIOS/FW Version | 0601 |
| CPU: | Ryzen 2700X |
| Memory: | G.Skill Trident Z CL14 F4-3200C14Q-64GTZ (64 GB) |
| HDD: | Samsung 950 Pro 1 TB |
| VGA: | NVidia Quadro NVS 295 |
| Power Supply: | Seasonic X-650 |
| Network/Lan: | (onboard device only) |
| Other Hardware: |  |
| OS Version: | Debian GNU/Linux, kernel version 4.16.5-1 |
| Related Driver(s): |  |

Troubleshooting steps:

To reproduce: 1. Load 0207 or 0401 BIOS; boot to OS and observe that PCIe devices are detected as expected. For a simple test, run: $ lspci ¦ grep -i eth 05:00.0 Ethernet controller: Intel Corporation I211 Gigabit Network Connection (rev 03) 2. Load 0509 or 0601 BIOS; boot to OS and re-run test: $ lspci ¦ grep -i eth There is no output from this, since the Ethernet device (and others) are not detected.